Analytical Error Debugging Assignment

Topic: Quantization Noise Power in an 8-bit ADC

Name:N.Algipran
Reg.no:192512494
Course name&Code:ECA0901-Digital Signal processing

1. Problem Statement (Assigned Task)

An 8-bit ADC with a ±5 V input range reports a quantization noise power of 0.01 V². Verify the calculation and identify the error.

2. Error Analysis — Identifying the Implementation Error(s)

Recomputing the quantization noise power from first principles (full derivation in Section 4) gives:

Pq (correct) = 1.2716 × 10⁻⁴ V²  ≈  127.16 µV²

This means the reported value (0.01 V²) is approximately 78.64× larger than the true value — far beyond rounding, indicating a genuine implementation/formula error. Two implementation errors are typically responsible for a discrepancy of this size:

| # | Implementation Error | Effect on Result |
| --- | --- | --- |
| E1 | Missing the ÷12 factor in the noise-power formula — using Pq = Δ² directly instead of Pq = Δ²/12 | Inflates Pq by exactly 12× |
| E2 | Using the bit count N (=8) instead of the level count 2^N (=256) when computing the step size — i.e. Δ = FSR/N instead of Δ = FSR/2^N | Inflates Δ by 32×, and Pq (∝ Δ²) by ≈1024× |

A variant / combination of E1 and E2 (e.g. an under-resolved level count such as 2⁴ or 2⁵ instead of 2⁸) reproduces a noise power in the 0.01 V² neighbourhood — consistent with the erroneous figure given in the problem.

3. Root Cause Analysis

Correct model of an ADC quantizer:

Δ  = FSR / 2^N        (quantization step size, "1 LSB")

Pq = Δ² / 12          (noise power, assuming the error e[n] = x[n] − xq[n]

is uniformly distributed over [−Δ/2, +Δ/2])

The discrepancy traces back to three possible root causes, of which the first two are the dominant contributors:

- Conceptual confusion between resolution (bits) and levels (2^N): an N-bit ADC has 2^N discrete output codes, not N. Substituting N directly for 2^N is a very common beginner error because both symbols intuitively “look like” the resolution of the converter.

- Dropping the 1/12 factor: this term comes from the variance of a uniform random variable on [−Δ/2, Δ/2]: Var = ∫ e²·(1/Δ) de over that range = Δ²/12. Reporting Δ² alone skips this statistical averaging step and reports the squared peak step instead of the average error power — these are physically different quantities.

- Range handling: FSR must use the full span (Vmax − Vmin = 10 V for ±5 V), not just one polarity (5 V). This was checked and ruled out as the dominant cause here since it only accounts for a 4× error in Pq, not 78×.

4. Required Calculations (Verification)

Given: Resolution N = 8 bits; Range Vmin = −5 V, Vmax = +5 V.

Correct derivation

FSR = Vmax − Vmin = 5 − (−5) = 10 V

L   = 2^N = 2^8 = 256 levels

Δ   = FSR / L = 10 / 256 = 0.0390625 V

Pq  = Δ² / 12 = (0.0390625)² / 12

= 0.00152587890625 / 12

= 1.2716 × 10⁻⁴ V²

Comparison

| Quantity | Claimed | Correct | Ratio (claimed / correct) |
| --- | --- | --- | --- |
| Quantization noise power Pq | 0.01 V² | 1.2716 × 10⁻⁴ V² | ≈ 78.64× |

This was independently confirmed with a Monte-Carlo simulation (1,000,000 uniformly distributed samples across the ±5 V range, quantized with an 8-bit / 256-level quantizer): the sample variance of the resulting quantization error matched Δ²/12, not 0.01 V².

MATLAB Verification: Code and Simulation Output

The Monte-Carlo result above was generated with the following MATLAB script. It computes Pq analytically from first principles, draws 1,000,000 uniformly distributed samples and quantizes them to confirm the sample variance matches Δ²/12, cross-checks the result against the standard SQNR formula, and plots the resulting quantization error distribution.

Coding:

%% ADC Quantization Noise Analysis - Verification Script

% Verifies the claimed quantization noise power (0.01 V^2) for an

% 8-bit ADC with +/-5 V full-scale range using a closed-form

% calculation, a Monte-Carlo simulation, and an SQNR cross-check.

clear; clc; close all;

%% Given parameters

N    = 8;               % ADC resolution (bits)

Vmin = -5;               % V

Vmax =  5;               % V

%% Closed-form (correct) calculation

FSR   = Vmax - Vmin;         % Full-scale range (V)

L     = 2^N;                 % Number of quantization levels

delta = FSR / L;             % Quantization step size (V)

Pq_correct = delta^2 / 12;   % Quantization noise power (V^2)

fprintf('FSR   = %.4f V\n', FSR);

fprintf('L     = %d levels\n', L);

fprintf('delta = %.10f V\n', delta);

fprintf('Pq (correct) = %.6e V^2\n', Pq_correct);

%% Compare against the claimed value

Pq_claimed = 0.01;                    % V^2, as given in the problem

ratio = Pq_claimed / Pq_correct;

fprintf('Pq (claimed) = %.4f V^2\n', Pq_claimed);

fprintf('Claimed is %.2fx too large\n', ratio);

%% Monte-Carlo verification

numSamples = 1e6;

x  = Vmin + (Vmax - Vmin) * rand(numSamples, 1);  % uniform input

xq = delta * floor(x/delta + 0.5);                 % quantize

xq = min(max(xq, Vmin), Vmax - delta);              % clip to range

err = x - xq;                                       % quantization error

Pq_sim = var(err);

fprintf('Pq (Monte-Carlo) = %.6e V^2\n', Pq_sim);

%% SQNR cross-check

SQNR_theory = 6.02*N + 1.76;             % dB

Ps = Vmax^2 / 2;                          % full-scale sine power

SQNR_calc = 10*log10(Ps / Pq_correct);

fprintf('SQNR (theory)    = %.2f dB\n', SQNR_theory);

fprintf('SQNR (corrected) = %.2f dB\n', SQNR_calc);

%% Plot the quantization error distribution

figure;

histogram(err, 100, 'Normalization', 'pdf');

xlabel('Quantization error (V)');

ylabel('Probability density');

title('Quantization error distribution (theory: uniform on \pm\delta/2)');

grid on;

Output — Simulated Quantization Error Distribution

Figure 1. Histogram of the simulated quantization error (1,000,000 samples) is flat across ±Δ/2 = ±0.01953 V, matching the theoretical uniform distribution and confirming Pq = Δ²/12 = 1.2716×10⁻⁴ V² — not the claimed 0.01 V².

5. Corrected Solution

Corrected quantization noise power:  Pq = Δ²/12 = 1.2716 × 10⁻⁴ V² (≈ 127.16 µV²)

Corrected step size:  Δ = 0.0390625 V (≈ 39.06 mV, i.e. 1 LSB)

The originally reported value of 0.01 V² should be discarded and replaced with 1.2716 × 10⁻⁴ V².

6. Justification Using DSP Concepts

6.1 Quantization as a nonlinear, memoryless operation

An ADC maps a continuous amplitude x[n] to the nearest of 2^N discrete levels spaced Δ apart. The number of representable levels scales exponentially with the bit count (2^N), not linearly — this is the defining property of binary encoding and is why substituting N for 2^N (Error E2) is a fundamental misunderstanding of the ADC model, not just an arithmetic slip.

6.2 Quantization error as additive uniform noise

For signals that are “busy” relative to the step size (spanning many levels, non-repetitive), the quantization error e[n] = x[n] − xq[n] is well modeled in DSP theory as a random variable uniformly distributed on [−Δ/2, Δ/2]. Its power (variance) is by definition Pq = E[e²] = ∫ e² (1/Δ) de over [−Δ/2, Δ/2] = Δ²/12. Reporting Δ² instead (Error E1) confuses the peak-to-peak step size squared with the statistical average power of the error — two different physical quantities related by the factor of 12 that comes directly from integrating over the uniform PDF.

6.3 Consistency check via Signal-to-Quantization-Noise Ratio (SQNR)

The standard DSP relation for a full-scale sinusoidal input is SQNR(dB) = 6.02·N + 1.76 dB. For N = 8, this gives ≈ 49.92 dB. Using the corrected Pq = 1.2716×10⁻⁴ V² with a full-scale sine of amplitude 5 V (signal power = 5²/2 = 12.5 V²) gives:

SQNR = 10·log10(12.5 / 1.2716×10⁻⁴) ≈ 49.92 dB

This matches the textbook formula exactly, confirming the corrected value is right. The erroneous Pq = 0.01 V² would instead predict SQNR = 10·log10(12.5/0.01) ≈ 31.0 dB, which does not match the known 8-bit SQNR relation — itself independent evidence that 0.01 V² is wrong.

6.4 Practical significance in a DSP system

Quantization noise power directly sets the noise floor of an ADC and, downstream, the achievable dynamic range of any DSP chain built on it. Overestimating Pq by ≈79× (as in the erroneous report) would lead a design to believe it has roughly 18 dB less SQNR margin than it actually has — potentially causing an engineer to over-specify resolution, oversample unnecessarily, or misjudge whether the ADC meets a system noise budget.

7. Conclusion

| Item | Value |
| --- | --- |
| Reported (erroneous) Pq | 0.01 V² |
| Corrected Pq | 1.2716 × 10⁻⁴ V² (≈ 127.16 µV²) |
| Error factor | ≈ 78.64× too large |
| Root cause | Using bit-count N instead of level-count 2^N, and/or omitting the ÷12 averaging factor in Pq = Δ²/12 |
| Verification method | Closed-form calculation + Monte-Carlo simulation (1×10⁶ samples) + SQNR cross-check (6.02N+1.76 dB rule) |