DSP EXPERIMENTS

NAME:N.ALGIPRAN

COURSE NAME & CODE:ECA0901-DIGITAL SIGNAL PROCESSING

5.1 Generate the coefficients of an FIR High-Pass filter using Rectangular

windowing techniques.

CODE:

clc; clear; close all;

%% --- Input Parameters ---

fp = 1000; % High-pass cutoff frequency (Hz)

fs = 8000; % Sampling frequency (Hz)

N = 50; % Filter order

wc = fp/(fs/2); % Normalized cutoff (0–1)

%% --- 1. FIR High Pass using Rectangular Window ---

b_rect = fir1(N, wc, 'high', rectwin(N+1));

disp('--- High-Pass Coefficients (Rectangular Window) ---');

disp(b_rect');

%% --- 2. FIR High Pass using Hamming Window ---

b_hamm = fir1(N, wc, 'high', hamming(N+1));

disp('--- High-Pass Coefficients (Hamming Window) ---');

disp(b_hamm');

%% --- 3. FIR High Pass using Hanning Window ---

b_hann = fir1(N, wc, 'high', hanning(N+1));

disp('--- High-Pass Coefficients (Hanning Window) ---');

disp(b_hann');

%% --- Frequency Response Plots ---

figure;

freqz(b_rect,1);

title('High-pass FIR using Rectangular Window');

figure;

freqz(b_hamm,1);

title('High-pass FIR using Hamming Window');

figure;

freqz(b_hann,1);

title('High-pass FIR using Hanning Window');

OUTPUT:

5.2. Generate the coefficients of an FIR High-Pass filter using Hamming windowing

Techniques

CODE:

clc; clear; close all;

%% --- Input Parameters ---

fp = 1000; % High-pass cutoff frequency (Hz)

fs = 8000; % Sampling frequency (Hz)

N = 50; % Filter order

wc = fp/(fs/2); % Normalized cutoff (0–1)

%% --- FIR High Pass using Hamming Window ---

b_hamm = fir1(N, wc, 'high', hamming(N+1));

disp('--- High-Pass Coefficients (Hamming Window) ---');

disp(b_hamm');

%% --- Frequency Response Plots ---

figure;

freqz(b_hamm,1);

title('High-pass FIR using Hamming Window');

title('High-pass FIR using Hanning Window');

OUTPUT:

5.3. Generate the coefficients of an FIR High-Pass filter using Hanning windowing

Techniques

CODE:

clc; clear; close all;

%% --- Input Parameters ---

fp = 1000; % High-pass cutoff frequency (Hz)

fs = 8000; % Sampling frequency (Hz)

N = 50; % Filter order

wc = fp/(fs/2); % Normalized cutoff (0–1)

%% --- FIR High Pass using Hanning Window ---

b_hann = fir1(N, wc, 'high', hanning(N+1));

disp('--- High-Pass Coefficients (Hanning Window) ---');

disp(b_hann');

%% --- Frequency Response Plots ---

figure;

freqz(b_hann,1);

title('High-pass FIR using Hanning Window');

OUTPUT: