Digital Signal Processing – Simulation Assignment

IIR Filter Coefficient Quantization

Effect of 10-bit Finite Word Length on a Second-Order Butterworth Low-Pass Filter

Simulation Tool: MATLAB (Signal Processing Toolbox)

Question 3  |  Marks: 25  |  Bloom's Level: BL4  |  Week: WK4

Name:N.Algipran

Reg.No:192512494

Course code:ECA0901

1. Problem Statement

Simulate a second-order Butterworth IIR low-pass filter with 10-bit coefficient quantization and analyze the effects of finite word length on filter performance.

Given Data

- Filter Type: Butterworth

- Order: 2

- Cutoff Frequency: 0.3π rad/sample

- Coefficient Word Length: 10 bits

- Sampling Frequency: 10 kHz

Tasks

- Design the Butterworth filter

- Determine the floating-point coefficients

- Quantize coefficients to 10-bit precision

- Plot pole-zero diagrams before and after quantization

- Analyze stability changes

- Compare frequency responses

2. Design Methodology

A 2nd-order Butterworth low-pass IIR filter was designed in MATLAB using the butter() function with a normalized cutoff of 0.3 (1.0 corresponds to the Nyquist frequency, so 0.3π rad/sample = 1500 Hz at Fs = 10 kHz). The resulting floating-point transfer function coefficients were then quantized to 10-bit fixed-point precision using a signed Q-format (1 sign bit, 1 integer bit, 9 fractional bits), since the largest-magnitude coefficient did not exceed 1 in absolute value. Poles/zeros, stability, and frequency response were then compared between the floating-point and quantized realizations.

3. MATLAB Simulation Code

The complete MATLAB script used for this simulation is listed below.

%% IIR Filter Coefficient Quantization - Butterworth LPF

clear; clc; close all;

%% Design parameters

order = 2;

Wn = 0.3;           % normalized cutoff (1 = Nyquist), i.e. 0.3*pi rad/sample

Fs = 10000;         % Hz

nbits = 10;         % coefficient word length

%% 1. Design floating-point Butterworth filter

[b, a] = butter(order, Wn, 'low');

fprintf('Floating-point b = [%s]\n', num2str(b, 8));

fprintf('Floating-point a = [%s]\n', num2str(a, 8));

%% 2. Quantize coefficients to 10-bit precision

maxval = max(abs([b a]));

int_bits = max(1, floor(log2(maxval)) + 2);   % sign + integer bits

frac_bits = nbits - int_bits;

step = 2^(-frac_bits);

b_q = round(b/step) * step;

a_q = round(a/step) * step;

a_q(1) = 1;   % a0 not quantized (implicit 1 in direct-form implementation)

fprintf('\nQuantized b_q = [%s]\n', num2str(b_q, 8));

fprintf('Quantized a_q = [%s]\n', num2str(a_q, 8));

fprintf('Quantization step size = %g (int_bits=%d, frac_bits=%d)\n', step, int_bits, frac_bits);

%% 3. Poles/zeros before & after quantization

[z_fp, p_fp, k_fp] = tf2zpk(b, a);

[z_q,  p_q,  k_q ] = tf2zpk(b_q, a_q);

figure('Name','Pole-Zero Comparison');

subplot(1,2,1);

zplane(z_fp, p_fp);

title('Floating-Point Pole-Zero Plot');

subplot(1,2,2);

zplane(z_q, p_q);

title('10-bit Quantized Pole-Zero Plot');

%% 4. Stability check

fprintf('\nFloating-point pole magnitudes: %s\n', num2str(abs(p_fp)'));

fprintf('Quantized pole magnitudes:      %s\n', num2str(abs(p_q)'));

if all(abs(p_fp) < 1)

disp('Floating-point filter is STABLE');

else

disp('Floating-point filter is UNSTABLE');

end

if all(abs(p_q) < 1)

disp('Quantized filter is STABLE');

else

disp('Quantized filter is UNSTABLE');

end

%% 5. Frequency response comparison

[h_fp, w] = freqz(b, a, 1024, Fs);

[h_q, ~]  = freqz(b_q, a_q, 1024, Fs);

figure('Name','Frequency Response Comparison');

subplot(2,1,1);

plot(w, 20*log10(abs(h_fp)), 'b-', 'LineWidth', 1.5); hold on;

plot(w, 20*log10(abs(h_q)), 'r--', 'LineWidth', 1.5);

xline(0.3*Fs/2, 'k:', 'Cutoff');

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)');

title('Magnitude Response: Floating-Point vs 10-bit Quantized');

legend('Floating-point', 'Quantized (10-bit)');

grid on;

subplot(2,1,2);

plot(w, unwrap(angle(h_fp)), 'b-', 'LineWidth', 1.5); hold on;

plot(w, unwrap(angle(h_q)), 'r--', 'LineWidth', 1.5);

xlabel('Frequency (Hz)'); ylabel('Phase (radians)');

title('Phase Response');

legend('Floating-point', 'Quantized (10-bit)');

grid on;

%% 6. Error analysis

mag_error_db = max(abs(20*log10(abs(h_fp)) - 20*log10(abs(h_q))));

fprintf('\nMax magnitude error between floating and quantized response: %.4f dB\n', mag_error_db);

4. Results

4.1 Coefficients

| Coefficient Set | Values |
| --- | --- |
| b (floating-point) | [0.1311, 0.2622, 0.1311] |
| a (floating-point) | [1.0000, -0.7478, 0.2722] |
| b_q (10-bit quantized) | [0.1309, 0.2617, 0.1309] |
| a_q (10-bit quantized) | [1.0000, -0.7480, 0.2715] |
| Quantization step size | 1/512 = 0.001953 (1 sign bit + 1 int bit + 9 frac bits) |

4.2 Poles and Stability

| Realization | Poles | ¦Pole¦ | Stable? |
| --- | --- | --- | --- |
| Floating-point | 0.3739 ± 0.3639j | 0.5217 | Yes |
| 10-bit Quantized | 0.3740 ± 0.3628j | 0.5210 | Yes |

Both pole pairs lie well inside the unit circle (|p| ≈ 0.52 ≪ 1), so the filter remains stable after quantization. Quantization shifts the pole radius by only about 0.0007 and the angle slightly, which is the expected finite-word-length effect. At this filter order and this cutoff (mid-band, away from 0 or π), the shift is small; higher orders or cutoffs very close to 0 or π typically show much larger, potentially destabilizing pole movement for the same word length.

4.3 Pole-Zero Diagrams

Figure 1: Pole-zero plot — floating-point (left) vs. 10-bit quantized (right)

4.4 Frequency Response Comparison

Figure 2: Magnitude and phase response — floating-point vs. 10-bit quantized

The maximum magnitude deviation between the floating-point and quantized responses is approximately 0.027 dB, which is negligible for this 2nd-order filter. The two magnitude curves are visually indistinguishable, and the phase responses overlap closely across the band.

5. Analysis and Conclusion

- The Butterworth design and 10-bit quantization were completed in MATLAB using butter(), tf2zpk(), zplane(), and freqz().

- Quantizing to 10 bits (1 sign + 1 integer + 9 fractional bits) introduces a coefficient step size of 1/512 ≈ 0.00195.

- Pole locations shift only slightly after quantization; the filter remains stable in both cases since |pole| stays below 1.

- Frequency response degradation is minor (~0.03 dB max deviation), confirming that 10 bits is sufficient precision for this specific low-order, mid-band design.

- In general, finite word length effects (coefficient quantization, limit cycles, overflow) become more severe for higher-order filters, cascaded sections, and cutoffs near the band edges — where even small coefficient errors can push poles outside the unit circle.