DSP EXPERIMENTS

NAME:N.ALGIPRAN

COURSE NAME & CODE:ECA0901-DIGITAL SIGNAL PROCESSING

| Expt 11 | IIR Filters using DSP Processor |
| --- | --- |
|  | 11.1 Design and Implementation of an IIR Low-Pass Filter using DSP Processor |

Objectives

- To design an IIR Low-Pass Filter with the given filter specifications.

- To implement the designed IIR Low-Pass Filter on the DSP processor (TMS320C5416 Starter Kit).

- To apply an input signal to the filter and obtain the filtered output using the DSP processor.

- To analyze and verify the performance of the IIR Low-Pass Filter by comparing the input and output responses.

Hardware and Software Required:

DSP – TMS320VC5416-5416 Board

Cpu

ME Universal Debugger DSP - 5416

Procedure:

- Connect the serial port cable between the PC and the VSK-5416 kit.

- Switch ON the VSK-5416 kit and press the RESET button.

- Open Vi Universal Debugger.

- Select Serial → Port Settings → Auto Detect and verify that the message "Vi Universal Debugger found in COMx" appears.

- Create a new project (File → New Project) and assign a project name.

- Create a new source file (File → New File) and write the program.

- Save the source file.

- Add the source file to the project.

- Add the command file (Micro5416.cmd) to the project.

- Save the project.

- Build/Rebuild the project and ensure there are no errors.

- Download the executable file to the VSK-5416 kit.

- Run the program.

- Observe and verify the output.

**************************************************************************

LOW PASS FILTER (IIR)

;***************************************************************************

*	butterworth filter

* IIR Filter, Created by Filter Design Package,

* (c) 2005, Vi Microsystems Pvt. Ltd.,

* Approximation Type: ButterWorth

* Filter Type: Low Pass Filter

- Filter Order: 2

- Sampling Frequency: 62

- Cutoff frequency in KHz = 4 .000000

;Variable declaration for the filter

| XN | .SET | 10H |
| --- | --- | --- |
| XNM1 | .SET | 11H |
| XNM2 | .SET | 12H |
| YN | .SET | 13H |
| YNM1 | .SET | 14H |
| YNM2 | .SET | 15H |
| SUM | .SET | 16H |
| ; Filter coefficient A1 .SET |  | Assignme t 0E903H |
| A2 | .SET | 0905H |
| B0 | .SET | 082H |
| B1 | .SET | 0104H |
| B2 | .SET | 082H |

.mmregs

.text START:

| STM | #0140H,ST0 | ; data page initialization to page 140 |
| --- | --- | --- |
| RSBX | CPL | ; reset the compiler mode bit |
| RSBX | FRCT | ; reset the fractional mode bit |
| NOP |  | ; wait for the reset action |
| NOP |  |  |
| NOP |  |  |

;initialize x(n),x(n-1),x(n-2),y(n),y(n-1),y(n-2)

; make all the variables to initially zero ST	#0H,XN

ST	#0H,XNM1

ST	#0H,XNM2

ST	#0H,YN

ST	#0H,YNM1

ST	#0H,YNM2 REPEAT:

| PORTR | 6,0 | ; give the start of conversion for adc |
| --- | --- | --- |
| nop |  | ; wait the some delay to conversion |
| nop |  |  |
| nop |  |  |
| nop |  |  |
| PORTR | 4,0 | ; read the sampled data from the port |
| LD | 0,A | ; load it the accumulator |
| AND | #0FFFH,A | ; extract the 12 bit alone |
| XOR | #0800H,A | ; to correct 2's complement |
| SUB | #0800H,A | ; convert the data to processor format |

| STL | A,XN | ; store the sampled data to XN |
| --- | --- | --- |
| LD | #0H,B | ; initialize the B reg to 0 for result |
| LD | XN,A | ; get the XN to accumulator |
| STLM | A,T | ; load it to the t reg |
| MPY | #B0,A | ; multiply the b0 coeff to the accumulator |
| LD | A,B | ; load the content of B reg to accumulator |
| LD | XNM1,A | ; get the XNM1 to accumulator |
| STLM | A,T | ; load it to T reg |
| MPY | #B1,A | ; multiply the b0 coeff to the accumulator |
| ADD | A,B | ; sum the current output with the B reg |
| LD | XNM2,A | ; get the XNM2 to accumulator |
| STLM | A,T | ; store the content of A reg to T reg |
| STLM | A,T | ; multiply the b2 coeff to the accumulator |
| MPY | #B2,A | ; sumup the current output with the B reg |
| ADD | A,B |  |
| LD | YNM1,A | ; get the YNM1 to accumulator |
| STLM | A,T | ; store the content of A reg to T reg |
| MPY | #A1,A | ; multiply the A1 coeff to the accumulator |
| SUB | A,B | ; subtract the current output from the B reg |
| LD | YNM2,A | ; get the YNM2 to accumulator |
| STLM | A,T | ; store the content of A reg to T reg |
| MPY | #A2,A | ; multiply the A1 coeff to the accumulator |
| SUB | A,B | ; subtract the current output from the B reg |
| SFTL | B,-12 | ; the resultant value in the B reg is shifted |
| STL | B,YN | ; right by 12 bits store the final output to |
| LD | YNM1,A | ; the variable YN exchange YNM1 to YNM2 |
| STL | A,YNM2 |  |
| LD | YN,A | ; exchange YN to YNM1 |
| STL | A,YNM1 |  |
| LD | XNM1,A | ; exchange XNM1 to XNM2 |
| STL | A,XNM2 |  |
| LD | XN,A | ; exchange XN to XNM1 |
| STL | A,XNM1 |  |
| LD | YN,A | ; load the final output data to accumulator |
| ADD | #800H,A | ; add the offset value |
| STL | A,YN | ; store the result in YN |
| PORTW | YN,04H | ; out the output to the DAC |
| B | REPEAT | ; branch to continue |

Output:

Result

- The IIR Low-Pass Filter was successfully designed according to the given specifications.

- The designed filter was implemented on the TMS320C5416 DSP Starter Kit.

- The input signal was processed through the IIR Low-Pass Filter, and the filtered output was obtained.

- The output confirmed that low-frequency components were passed while high-frequency components were attenuated, verifying the correct operation of the filter.